Problem Set 3: EE 145B: Sharmila Majumdar

- Using Fox+noise 1 (from the website) and Fox + Noise 2, demonstrate the following in Mathlab:

- Histogram equalization of both images using 16 and 64 bins respectively. The functions used should include histeq and imhist. Display images using imagesc

- Manipulate the image such that all gray level values lie between 0 and 1. Then threshold the image so that the lower 20% is mapped to 0 and the rest to 1. Compare the results of doing this transformation on the two fox images.

2. A sinogram is the projection of an object as seen from the angles

0 to 180 degrees with a fixed increment.

Create a phantom array 128 x 128, where all values are zero, except for the presence of a central "plus sign" 10 high and 10 wide, each line being 5 pixels such as this:

10

5

10

At 10 degree increments, create a sinogram. Use this sinogram to recreate the image using the FFT based reconstruction technique.

Recreate the sinogram at 2 degree increments, and reconstruct the image again.

Use a Ramp filter and recreate the image.

3. Construct a phantom that is circular, inner radius 7 outer radius 20:

2

10

Embed it in a 128 x 128 array. Obtain 4 projections, and reconstruct the image. Obtain 20 projections and recreate the image, and final obtain 180 projections and recreate the image.

4. A material has half the attenuation coefficient of bone.For the same thickness of material, how will the intensity of the emergent xray beams be related?